2MP Fixed Dual-light Turret Network Camera IPC3612LB-AF28-DL-ECO

2MP Fixed Dual-light Turret Network Camera                                                IPC3612LB-AF28-DL-ECO2

Features

- High quality image with 1080P, 1/3.0" CMOS sensor

- 1080P (1920 × 1080)@30/25 fps; 720P (1280 × 720)@30/25 fps

- Smart dual-light ensures high quality image

- Supports accurate human body detection 2.0

- Ultra 265, H.265, H.264

- 2D/3D DNR (digital noise reduction)

- ROI (region of interest) ensures image quality in areas of interest with low bandwidth

- Smart IR, up to 30 m (98.4 ft.) IR distance

- Warm light, up to 15 m (49.2 ft.) warm light distance

- Built-in mic

- Wide temperature range: -30 ℃ to 60 ℃ (-22 ℉ to 140 ℉)

- Supports PoE power supply

- IP67

Specifications

| Model | IPC3612LB-AF28-DL-ECO2 |
| --- | --- |
| Camera |  |
| Max Resolution | 2 MP |
| Sensor | 1/3.0" CMOS |
| Min. Illumination | Color: 0.005 Lux (F1.6, AGC ON), 0 Lux with IR on |
| Day/Night | IR-cut filter with auto switch (ICR) |
| Shutter | Auto/Manual, 1 to 1/100000 s |
| Adjustment Angle | Pan: 0° to 360°, Tilt: 0° to 80°, Rotate: 0° to 360° |
| S/N | > 52 dB |
| WDR | DWDR |
| Lens |  |
| Focal Length | 2.8 mm |
| Iris | F1.6 |
| Field of View (H) | 85.1 ° |
| Field of View (V) | 46.6 ° |
| Field of View (D) | 95.9 ° |
| Lens Type | Fixed |
| Iris Type | Fixed |
| DORI |  |
| DORI Distance (Lens) | 2.8 mm |
| DORI Distance (Detect) | 51.4 m (168.7 ft.) |
| DORI Distance (Observe) | 20.6 m (67.5 ft.) |
| DORI Distance (Recognize) | 10.3 m (33.8 ft.) |
| DORI Distance (Identify) | 5.1 m (16.9 ft.) |
| Illuminator |  |
| Illumination Distance (IR) | 30 m (98.4 ft.) |
| Illumination Distance (Warm Light) | 15 m (49.2 ft.) |
| Wavelength | 850 nm |
| IR On/Off Control | Auto/Manual |
| Supplemental Light | Dual light |
| Video |  |
| Video Compression | Ultra 265, H.265, H.264 |
| H.264 Code Profile | Baseline profile, Main profile, High profile |
| Frame Rate | Main Stream: 1080P (1920 × 1080), max. 30 fps Sub Stream: 640 × 360, max. 30 fps; 2CIF (704 × 288), max. 30 fps; CIF (352 × 288), max. 30 fps; |
| Video Bit Rate | 128 Kbps to 6144 Kbps |
| U-code | Support |
| OSD | Up to 4 OSDs |
| Privacy Mask | Up to 4 areas |
| Snapshot | N/A |
| ROI | Up to 8 areas |
| Video Stream | Dual streams |
| Image |  |
| Smart IR | Support |
| White Balance | Auto, Outdoor, Fine tune, Sodium lamp, Locked, Auto2 |
| Digital Noise Reduction | 2D/3D DNR |
| Flip | Normal, Flip vertical, Flip horizontal, 180° |
| Dewarping | N/A |
| HLC | Support |
| BLC | Support |
| Defog | Digital defog |
| Events |  |
| Basic Detection | Motion detection, Audio detection, Human body detection 2.0 |
| General Function | Watermark, IP address filtering, Access policy, ARP protection, RTSP authentication, User authentication, HTTP authentication |
| Audio |  |
| Audio Compression | G.711U, G.711A |
| Audio Bitrate | 64 Kbps |
| Two-way Audio | N/A |
| Suppression | Support |
| Sampling Rate | 8 kHz |
| Storage |  |
| Edge Storage | N/A |
| Network |  |
| Protocols | IPv4, IGMP, ICMP, ARP, TCP, UDP, DHCP, RTP, RTSP, DNS, DDNS, NTP, UPnP, HTTP, QoS, RTCP, RTMP |
| Compatible Integration | API, SDK |
| Client | EZStation, APP |
| Web Browser | Plug-in required live view: IE 10+, Chrome 45+, Firefox 52+, Edge 79+, Plug-in free live view: Chrome 57+, Firefox 58+, Edge 16+ |
| Security | Password protection, Strong Password, Export operation logs, Basic and Digest Authentication for RTSP, Digest Authentication for HTTP, WSSE and Digest Authentication for ONVIF |
| User/Host | Up to 32 users. 2 user levels: administrator and common user |
| Interface |  |
| Audio I/O | N/A |
| Alarm I/O | N/A |
| Serial Port | N/A |
| Built-in Mic | Support |
| Built-in Speaker | N/A |
| WIFI | N/A |
| Network | 1 × RJ45 10 M/100 M Base-TX Ethernet |
| Video Output | N/A |
| Certification |  |
| Protection | IP67 (IEC 60529) |
| EMC | CE-EMC (EN 55032, EN 61000-3-3, EN IEC 61000-3-2, EN 50130-4) FCC (FCC 47 CFR part15 B) |
| Safety | CE-LVD (EN 62368-1) |
| Environment | CE-RoHS (2011/65/EU ; (EU) 2015/863) ; WEEE (2012/19/EU) |
| General |  |
| Power | PoE (IEEE 802.3af) |
| Power Consumption | Max. 3.7 W |
| Power Interface | N/A |
| Dimensions | Ø117.8 mm × 100.1 mm (Ø4.6" × 3.9") (Ø × H) |
| Weight | 0.22 kg (0.49 lb) |
| Material | Graphene + Plastic |
| Working Environment | -30 ℃ to 60 ℃ (-22 ℉ to 140 ℉), Humidity: ≤ 95% RH (non-condensing) |
| Storage Environment | -30 ℃ to 60 ℃ (-22 ℉ to 140 ℉), Humidity: ≤ 95% RH (non-condensing) |
| Surge Protection | 4 KV |
| Reset Button | N/A |
| Web Client Language | 1 Language: English |

Dimensions

Accessories

| TR-JB03-I-IN | TR-A01-IN | TR-SE24-IN |
| --- | --- | --- |
| Junction Box for Turret Camera(Support wiring from behind) | NPT 3/4'' Waterproof Cable Gland | Dome Pendent Mounting Pole 200mm |
| TR-SE24-A-IN | TR-CM24-IN | TR-JB07/WM03-G-IN-V2 |
| Dome Pendent Mounting Pole 500mm | Dome Pendent Mounting Bracket(Indoor) | Wall Mounting Assembling Bracket For 32 Fix-dome&Mini Fix-dome&Plastic Turret Camera |
| TR-WM03-D-IN-V2 | TR-UC08-C-V2 | TR-UP06-C-IN-V2 |
| Wall Mounting Bracket for 32 Fix-dome&Mini Fix-dome&Plastic Turret Camera | Bullet&Dome Corner Mounting Bracket | Pole Mounting Bracket for Bullet Junction Box Only |
| TR-WM07P-B |  |  |
| Dome Wall Mounting Plastic Bracket |  |  |

| Unlimited New View |  |  |
| --- | --- | --- |
| Zhejiang Uniview Technologies Co., Ltd. |  |  |
|  | http://www.uniview.com |  |
|  | overseasbusiness@uniview.com; globalsupport@uniview.com |  |
|  | No. 369, Xietong Road, Xixing Sub-district, Binjiang District, Hangzhou City, 310051, Zhejiang Province, China |  |
| ©2026 Zhejiang Uniview Technologies Co., Ltd. All rights reserved. *Product specifications and availability are subject to change without notice. Despite our best efforts, technical or typographical errors may exist in this document. Uniview cannot be held responsible for any such errors and reserves the right to change the contents of this document without prior notice. |  |  |